國立嘉義大學99學年度
土木與水資源工程學系碩士班（乙組）招生考試試題

科目：流體力學
說明：1.如有條件不足之情形，請自行假設。2.僅可使用試務單位提供之計算機。
1. (a) Please derive the differential form of continuity equation (5％)
(b) What is the incompressible fluid﹖(write an equation to explain the definition of incompressible fluid). Please write the differential form of continuity equation for the incompressible fluid. (5％)
(c) If the velocity filed is 
[image: image3.png]\

Free jet



, determine the volumetric dilatation rate. Is the fluid in the flow filed compressible or incompressible? (10％)

2. A mountain lake has an average temperature of 10 °C and a maximum depth of 45 m. For a barometric pressure of 598 mm Hg, determine the absolute pressure (in pascals) at the deepest part of the lake. (the unit weight of water at 10°C is 9.804 kN/m3) (20％)

3. A certain spillway for a dam is 20 m wide and is designed to carry 125 m3/s at flood stage. A 1:15 model is constructed to study the flow characteristics through the spillway. Determine the required model width and flowrate. The effects of surface tension and viscosity are to be neglected. (20％)
4. As shown in Fig.1, at the entrance to 3-ft-wide channel the velocity distribution is uniform with velocity V. Further downstream the velocity profile is given by u = 4y-2y2, where u is in ft/s and y is in ft. Determine the value of V. (20％)
[image: image1.wmf]22

(,,)42()0

Vxyzxyixyjk

=+-+

urrrr


Fig.1
5. A free jet of fluid strikes a wedge as shown in Fig. 2, Of the total flow, a portion is deflected 30°; the remainder is not deflected. The horizontal and vertical components of force needed to hold the wedge stationary are FH and FV, respectively. Gravity is negligible, and the fluid speed remains constant. Determine the force ratio, FH/FV. (20％)
[image: image2.png]



Fig.2









_1296801817.unknown

